SMART FARMING ASSISTANT

Technical Requirements Document

System Architecture & Implementation Specification — Smart India Hackathon (SIH)

| Document Type | TRD |
| --- | --- |
| Project | Smart Farming Assistant |
| Companion Doc | PRD v1.0 |
| Prepared By | Divyadarshini M |
| Version | 1.0 |
| Date | September 2026 |

# 1. System Overview

Smart Farming Assistant follows an edge-first architecture: two field sensor nodes stream environmental data over a low-power wireless link to a central Edge AI Gateway, which performs local sensor processing, image preprocessing, disease/pest inference, sensor+vision fusion, and risk decision-making. A farmer's smartphone acts as the image acquisition device, connecting to the gateway over local Wi-Fi. Results and alerts are surfaced on a web/mobile dashboard, with cloud sync used only for historical storage and remote reporting when internet is available.

# 2. Architecture Diagram (Logical Flow)

Node 1 (Zone 1) ┐

Node 2 (Zone 2) ┘─ LoRa/Wireless → Edge AI Gateway

Farmer Smartphone ─ Local Wi-Fi → Edge AI Gateway

Edge AI Gateway ─ Internet (optional/best-effort) → Cloud Dashboard/Alerts

## 2.1 Component Responsibilities

| Component | Responsibility |
| --- | --- |
| Field Sensor Node (x2) | Soil moisture, temperature, humidity, rain, optional water-level sensing; periodic wireless transmission |
| Edge AI Gateway | Sensor data processing, image preprocessing, Disease AI, Pest AI, sensor+vision fusion, risk engine, local database, alert generation |
| Farmer Smartphone (App) | Image capture, zone tagging, local Wi-Fi upload to gateway, alert/dashboard viewing |
| Web Dashboard | Visualization, historical analytics, farmer advisory, settings, reporting |
| Cloud Backend (optional sync) | Long-term storage, multi-farm aggregation, remote access, push notifications |

# 3. End-to-End Data Flow (8 Stages)

| # | Stage | Description |
| --- | --- | --- |
| 1 | Field Monitoring (Continuous) | Nodes collect soil moisture, temperature, humidity, and rainfall data and transmit wirelessly to the gateway on a fixed interval. |
| 2 | Risk Detection & Image Trigger | Gateway analyzes sensor data per zone (e.g., low moisture + high temp → water stress; high humidity + recent rain → disease risk) and prompts the farmer to capture an image of the flagged zone. |
| 3 | Image Acquisition | Farmer opens the mobile app, selects the zone, and captures a close-up image; app tags location/zone ID, compresses the image, strips metadata, and sends it over local Wi-Fi. |
| 4 | Image Preprocessing (Edge) | Gateway performs quality check (blur/lighting), resize (e.g., 320×320), crop to region of interest, and normalization for model input. |
| 5 | AI Analysis | Disease AI and Pest AI models run inference on the preprocessed image, producing class probabilities (e.g., Blast 0.82, Brown Spot 0.08, Healthy 0.06) and a pest/disease confidence score. |
| 6 | Sensor + Image Fusion | Fusion engine combines image-based classification results with environmental sensor data (temperature, humidity, rainfall, soil) to compute a final, calibrated risk score. |
| 7 | Decision & Alert | Risk engine classifies risk (e.g., Zone 2 — Possible Blast, Confidence: Medium/High) and generates a recommended action set (inspect, check symptoms, follow crop-protection guidance). |
| 8 | Dashboard / Farmer Action | Alert and recommendation are pushed to the dashboard/app; farmer takes action in the field; outcome and treatment history are logged for trend analysis. |

# 4. Hardware Specification

## 4.1 Field Sensor Node

- Sensors: capacitive soil moisture sensor, DHT22/SHT-series temperature & humidity sensor, tipping-bucket or optical rain sensor, optional ultrasonic water-level sensor.

- Microcontroller: low-power MCU (e.g., ESP32) with LoRa module for long-range, low-power transmission to the gateway.

- Power: solar panel with battery backup for continuous field operation.

- Enclosure: weatherproof (IP65+) housing rated for outdoor agricultural conditions.

## 4.2 Edge AI Gateway

- Compute: single-board computer capable of local CNN inference (e.g., Raspberry Pi 4/5 or Jetson-class board for higher throughput).

- Connectivity: LoRa receiver (for nodes), local Wi-Fi access point (for farmer's phone), optional cellular/Wi-Fi uplink for cloud sync.

- Storage: local SSD/eMMC for buffering sensor data, images, and model artifacts during offline operation.

# 5. Software Architecture

## 5.1 Technology Stack

| Layer | Suggested Technology |
| --- | --- |
| Sensor Node Firmware | C/C++ (Arduino/ESP-IDF framework) |
| Edge Gateway Runtime | Python (edge services), Docker for service isolation |
| Edge AI Inference | TensorFlow Lite / ONNX Runtime for on-device Disease AI & Pest AI models |
| Edge Local Database | SQLite / TinyDB for offline-first local storage |
| Mobile App | React Native or Flutter (cross-platform image capture & alerts) |
| Web Dashboard Frontend | React.js with a component library and charting library (e.g., Recharts/Chart.js) |
| Backend / Cloud API | Node.js (Express) or Python (FastAPI) REST API |
| Cloud Database | PostgreSQL (structured data) + object storage (S3-compatible) for images |
| Messaging / Sync | MQTT or REST batch sync between gateway and cloud when connectivity is available |
| Weather Integration | Third-party weather API (e.g., OpenWeatherMap) for forecast and rain probability |

## 5.2 AI Models

- Disease Detection Model: CNN (e.g., MobileNetV2/EfficientNet-Lite backbone, transfer-learned) classifying leaf images into disease classes plus “Healthy” and “Others,” with an associated confidence score.

- Pest Detection Model: CNN or lightweight object-detection model identifying pest type, presence, and estimated activity/population level from leaf/trap images.

- Fusion/Risk Engine: rule-based or lightweight ML model combining image classification confidence with environmental features (temperature, humidity, rainfall, soil moisture) to produce a final risk level (Low/Medium/High) and confidence band.

- Model deployment: quantized (INT8) models converted to TensorFlow Lite/ONNX for real-time on-device inference within the gateway's compute budget.

# 6. Data Model (Core Entities)

| Entity | Key Fields |
| --- | --- |
| Farm | farm_id, name, location, owner, created_at |
| Field / Zone | zone_id, farm_id, boundary_geo, crop_type, growth_stage |
| SensorNode | node_id, zone_id, node_type, last_seen, battery_status |
| SensorReading | reading_id, node_id, timestamp, soil_moisture, temp, humidity, rainfall, water_level |
| ImageCapture | image_id, zone_id, captured_by, timestamp, storage_url, preprocessing_status |
| DetectionResult | result_id, image_id, model_type (disease/pest), class_label, confidence, severity |
| RiskAssessment | risk_id, zone_id, timestamp, risk_type, risk_level, contributing_factors, fusion_score |
| Alert | alert_id, zone_id, severity, category, message, recommended_action, status, created_at |
| TreatmentRecommendation | rec_id, risk_id, active_ingredient, product, dosage, method, timing, cost_estimate |
| Dealer | dealer_id, name, location, distance, contact, product_stock |
| TreatmentHistory | history_id, zone_id, treatment_date, product_used, pre/post condition, effectiveness |

# 7. API Design (Representative Endpoints)

GET  /api/v1/farms/{farmId}/dashboard

GET  /api/v1/zones/{zoneId}/sensors/latest

POST /api/v1/zones/{zoneId}/images            (multipart image upload)

GET  /api/v1/zones/{zoneId}/detections

GET  /api/v1/zones/{zoneId}/irrigation/recommendation

GET  /api/v1/zones/{zoneId}/advisory

GET  /api/v1/alerts?severity=critical&status=active

GET  /api/v1/dealers?product={productId}&near={lat,lng}

GET  /api/v1/analytics/{zoneId}/trends?metric=soil_moisture&range=30d

# 8. Communication Protocols

| Link | Protocol | Notes |
| --- | --- | --- |
| Sensor Node ↔ Gateway | LoRa (sub-GHz) | Long range, low power; suited to open farmland with sparse infrastructure |
| Farmer Phone ↔ Gateway | Local Wi-Fi (HTTP/REST) | Enables image upload and dashboard access without internet dependency |
| Gateway ↔ Cloud | HTTPS REST / MQTT | Best-effort batch sync; queues data locally when offline |
| Dashboard ↔ Backend | HTTPS REST, WebSocket (for live alerts) | WebSocket used for real-time alert push to connected clients |

# 9. Non-Functional Requirements

| Category | Requirement |
| --- | --- |
| Performance | Edge inference latency < 3 seconds per image; sensor-to-alert latency < 5 minutes |
| Availability | Core monitoring/alerting functions operate fully offline; ≥ 99% local uptime |
| Scalability | Gateway supports ≥ 8 sensor nodes and multiple concurrent image uploads per deployment |
| Security | TLS for all cloud sync traffic; local Wi-Fi secured with WPA2/WPA3; role-based access on dashboard |
| Data Privacy | Farm and location data isolated per farmer/cooperative account; images stored with access control |
| Power Efficiency | Sensor nodes designed for multi-month operation on solar + battery power |
| Usability | Dashboard and mobile app support low-literacy UI patterns (icons, color-coded risk, local language) |
| Maintainability | Modular microservice-style edge services (sensor, image, fusion, alert) independently updatable |

# 10. Deployment & Environment

- Development: local simulation of sensor data and sample leaf-image datasets for model training/testing before field deployment.

- Staging: single-node + single-gateway pilot deployment in a controlled field/greenhouse for validation.

- Production (Pilot): two-node deployment per field unit as described in Section 3, with dashboard hosted on a cloud provider (e.g., AWS/GCP/Azure free-tier for hackathon demo).

- CI/CD: containerized edge services (Docker) with version-tagged model artifacts for reproducible deployment to gateway hardware.

# 11. Risks & Mitigations

| Risk | Mitigation |
| --- | --- |
| Limited/no internet connectivity in the field | Edge-first design: all core detection/alerting runs locally without cloud dependency |
| Low AI model accuracy on unseen disease/pest classes | Start with a limited, well-labeled dataset for common regional crops; expand iteratively |
| Sensor node power failure | Solar + battery with low-power sleep cycles; battery status surfaced in dashboard |
| Farmer misinterprets AI confidence as certainty | UI clearly labels advisory outputs with confidence bands and recommends inspection before action |